	             
	ALS Inspection Chile SpA

 Limache 3405, Office 61
Viña del Mar, CHILE

T +56 32 2545 500
Report: ALS-2601-0078
Date: January 14, 2026



[image: image259.png]




	MV “LEO OCEAN”

	[image: image1.jpg]




	FULL OFF HIRE & BUNKER SURVEY


	This is to certify that the undersigned representative of ALS did at the request of Messrs. DAMPSKIBSSELSKABET NORDEN A/S attend on board motor vessel “LEO OCEAN” with the Master consent on January 11, 2026, as she was berthed at Pier 2-B of COSCO Shipping Ports Chancay Peru S.A., in Chancay port, Peru, in order to carry out an Off Hire, general condition survey and bunkers remaining onboard at time of re-delivery, as follows:


	COMMENTS BY SURVEYOR

	Vessel with certificates updated. Hull noted with several areas with paint stained and scratched, and with rust; superstructure in good condition. Cargo holds were observed empty; Fwd and Aft bulkhead with paint stains and scratched, and with bent steps ladders. Port and Starboard side shell plating with paint stained and scratched. Hatch covers in apparent good structural conditions. Cargo cranes were observed in apparent good conditions. Mooring ropes in poop deck and forecastle were found in good conditions.


INDEX
	Content 
	Page Nr.

	I.
	Times
	3

	II.
	Vessel’s particulars
	3

	III.
	Capacities
	3

	IV.
	Certificates Status
	4

	V.
	External Hull Condition
	5

	VI.
	General Statement of hull structure
	5

	VII.
	Deck
	10

	VIII.
	Holds & Hatches
	20

	IX.
	Cargo Gear Conditions
	56

	X.
	Bunker Survey
	61 & 62

	XI.
	Attending Parties
	63

	XII.
	Attachments
	63


TERMS AND Definitions of Condition      
	Good
	G
	Unimpaired condition without significant wear or deviation from original strength and operating efficiency. No maintenance or repair required.

	Fair


	F
	Condition in which wear and tear and other deficiencies of minor nature not requiring correction or repair.

	Poor


	P
	Condition in which the adequacy of strength and/or operational efficiency is marginally below acceptable limits, or is in doubt. Remedial action is required.

	Unsatisfactory


	U
	Condition of undoubtedly inadequate strength, or operational efficiency. Immediate extensive repair/renewal, required to reinstate serviceability.

	Observation
	Obs Nr.
	Indicate observation number (a special condition or important remark). 


SCOPE OF SURVEY
· To verify capacities and general arrangement plans, loading manuals and all certificates and to report upon them.

· To examine the weather deck and superstructures, cargo spaces including the cargo cranes, cargo gear and to record all defects.

· To examine the main hull/structures and fittings in each cargo spaces and to record all defects.

· To examine vessel's condition, cleanliness and load readiness and to record all defects.

· To establish by sounding and calculation the quantity of fuel on board at the time of survey and to establish the fuel consumption from the time the vessel became Off-Hire to the time of sounding.
I.  TIMES:
	Date
	Time
	Description

	January 11, 2026
	15:00
	ALS Surveyors arrival to Chancay Port.

	
	15:35
	Surveyor on board MV “LEO OCEAN”.

	
	15:50 to 04:45
	Inspection cargo holds Nr. 1, 2, 3, 4 & 5.

	
	16:00
	Bunker survey started.

	
	17:00
	Bunker survey completed.

	
	19:00 to 19:50
	Inspection to vessel’s certificates.

	January 12, 2026
	05:25
	ALS surveyors left the vessel.


II.   VESSEL’S PARTICULARS:
	Master Name
	Mr. Jose Mari L. Cabanatan

	Chief Officer Name
	Mr. Marlon C. Jalandoni 

	Chief Engineer
	Mr. Jess M. Amparo

	Flag & Port of Register
	Panamanian / Panama

	Registered Owners
	SUPER CAMELLIA S.A.

	Year Built
	2015

	Length Overall
	199,98 m.

	Breath 
	32,24 m

	Summer. Draft
	13,010 m.

	GRT
	34,800 tons. 

	NRT
	20,193 tons.

	Classification Society
	Nippon Kaiji Kyokai (NKK)

	Cargo gear
	4 set of cranes SWL-30.5 Tons x 26 meters


III.  CAPACITIES:

	Cargo Capacity
	Grain: 77,610.95 cbm. / Bale: 73,551.72 cbm.

	Summer DWT
	61,315.0 mt

	IFO / MDO
	2,156.48 cbm. / 409.05 cbm.

	Lub Oil
	17.75 cbm.

	Hydraulic Oil
	--

	Fresh Water
	669.12 cbm.

	Ballast Water 
	34,072.59 cbm.


IV. CERTIFICATES STATUS:  

	Certificate / Document
	Issue date
	Expiry date
	Endorsement

	
	Issue place
	
	

	Certificate of Registry
	January 16, 2024
	February 05, 2029
	Flag State

	
	Panama
	
	

	Certificate of Class
	August 11, 2025
	May 24, 2030
	NKK

	
	Zhoushan, China
	
	

	International Tonnage 
	December 12, 2018
	
	NKK

	
	New York
	
	

	Suez Canal Tonnage
	May 25, 2015
	
	NKK

	
	Hiroshima
	
	

	Panama Tonnage 
	April 12, 2017
	
	ACP PANAMA

	
	Balboa
	
	

	International load line
	August 11, 2025
	May 24, 2030
	NKK

	
	Zhoushan, China
	
	

	Cargo ship safety construction
	August 11, 2025
	May 24, 2030
	NKK

	
	Zhoushan, China
	
	

	Cargo Ship Safety Equipment
	August 11, 2025
	May 24, 2030
	NKK

	
	Zhoushan, China
	
	

	Annual Cargo Gear Examination
	May 04, 2025
	May 03, 2026
	NKK

	
	Guayaquil, Ecuador
	
	

	International Oil Pollution Prevention
	August 11, 2025
	May 24, 2030
	NKK

	
	Zhoushan, China
	
	

	Last Port State Control
	January 06, 2026
	-
	MGP

	
	Callao, Peru
	
	

	Last Dry Dock
	August, 2025
	-
	-

	
	Zhoushan, China
	
	


V. EXTERNAL HULL CONDITION:  
	
	G
	F
	P
	U
	Obs Nr

	Structural condition


	
[image: image2.wmf]
	
	
	
	
	

	
	
	
	
	
	
	

	Overall appearance


	
[image: image3.wmf]
	
	
	
	
	

	
	
	
	
	
	
	

	Coating condition


	
	
[image: image4.wmf]
	
	
	1
	

	
	
	
	
	
	
	


OBSERVATIONS:  

Nr. 1: Hull plating Port and Starboard side, with paint stained, scratched and with rust. (See photo Nr. 1, 2, 3, 4, 5, 6, 7, 8 & 9).
VI.   GENERAL STATEMENT OF HULL STRUCTURE:
	Upon viewing vessel from various angles, were there any apparent or obvious signs of major longitudinal or transverse deformation or structural failure which might indicate earlier serious damage or poor repairs?
	  Y
	  N

	
	
	
[image: image5.wmf]


	[image: image6.jpg]




	1.- Hull plating/Fwd Ps, with paint stained, scratched, and with rust.

	[image: image7.jpg]




	2.- Hull plating/ Fwd Ps & bulbous, with paint stained, scratched, and with rust.

	[image: image8.jpg]




	3.- Hull plating/ Ps, with paint stained, scratched, and with rust.

	[image: image9.jpg]




	4.- Hull plating/ Ps, with paint stained, scratched, and with rust.

	[image: image10.jpg]




	5.- Hull plating/ Ps, with paint stained and scratched.

	[image: image11.jpg]




	6.- Hull plating/ Ps, with paint stained, scratched, and with rust.

	[image: image12.jpg]




	7.- Hull plating/ Ps, with paint stained, scratched, and with rust.

	[image: image13.jpg]




	8.- Hull plating/ Aft Ps, with paint stained, scratched, and with rust.

	[image: image14.jpg]




	9.- Hull plating/ Aft Ps & Rudder, with paint stained, scratched, and with rust.


VII.   DECK: 

	FORECASTLE DECK
	G
	F
	P
	U
	Obs Nr
	POOP DECK
	G
	F
	P
	U
	Obs Nr

	Anchor equipment


	
[image: image15.wmf]
	
	
	
	
	
	Winches
	
[image: image16.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Railings and bulwark

	
[image: image17.wmf]
	
	
	
	
	
	Systems and bulkwark
	
[image: image18.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Air and sounding pipes
	
[image: image19.wmf]
	
	
	
	
	
	Air and sounding pipes
	
[image: image20.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Plating
	
[image: image21.wmf]
	
	
	
	
	
	Plating
	
[image: image22.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Mooring lines
	
[image: image23.wmf]
	
	
	
	
	
	Mooring lines
	
[image: image24.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Light mast
	
[image: image25.wmf]
	
	
	
	
	
	Access
	
[image: image26.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Bollards


	
[image: image27.wmf]
	
	
	
	
	
	Bollards


	
[image: image28.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Fair leaders
	
[image: image29.wmf]
	
	
	
	
	
	Fair leaders
	
[image: image30.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	


	PORT SIDE DECK
	G
	F
	P
	U
	Obs Nr
	STB SIDE DECK
	G
	F
	P
	U
	Obs Nr

	Hatchcoaming     
	
[image: image31.wmf]
	
	
	
	
	
	Hatchcoaming     
	
[image: image32.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Deck plating          
	
[image: image33.wmf]
	
	
	
	
	
	Deck plating          
	
[image: image34.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Railings / bulwark

	
[image: image35.wmf]
	
	
	
	
	
	Railings / bulwark

	
[image: image36.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Piping in way         
	
[image: image37.wmf]
	
	
	
	
	
	Piping in way         
	
[image: image38.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Air and 

sounding pipes
	
[image: image39.wmf]
	
	
	
	
	
	Air and 

sounding pipes
	
[image: image40.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Stevedore’s access    
	
[image: image41.wmf]
	
	
	
	
	
	Stevedore’s access    
	
[image: image42.wmf]
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	


OBSERVATIONS:  

NIL
	[image: image43.jpg]




	10.- View of forecastle deck plating.

	[image: image44.jpg]




	11.- Forecastle deck, anchor chain view.

	[image: image45.jpg]




	12.- Forecastle deck Ss, winch & mooring lines working.

	[image: image46.jpg]




	13.- Forecastle deck Ps, winch & mooring lines working.

	[image: image47.jpg]




	14.- Forecastle deck Ss. Fairlead roller view.

	[image: image48.jpg]




	15.- Forecastle deck Ss. Bollards view.

	[image: image49.jpg]




	16.- Forecastle deck, view of mooring lines condition.

	[image: image50.jpg]




	17.- View of emergency life raft in good condition.

	[image: image51.jpg]




	18.- View of Port side main deck.

	[image: image52.jpg]




	19.- View of Stbd side main deck.


	[image: image53.jpg]




	20.- View of Poop deck plating.

	[image: image54.jpg]




	21.- Poop deck, view of mooring lines condition.


	[image: image55.jpg]




	22.- Poop deck Ss, winch & mooring lines working.

	[image: image56.jpg]




	23.- Poop deck Ps, winch & mooring lines working.


	[image: image57.jpg]i e






	24.- Poop deck Ps. Fairlead roller view.

	[image: image58.jpg]




	

	25.- Poop deck Ss. Bollards view.


	[image: image59.jpg]




	26.- General view of Grabs N° 4.

	[image: image60.jpg]




	27.- General view of gangway ladder Ps, in good conditions.


VIII.    HOLDS & HATCHES:
	[image: image61.emf]Saddle or Top side tank


Estanques 


laterales superiores


Shell


Planchaje


 


Hopper tank


Estanques embudo 


Side girders


Vigas laterales 


Double bottom tank


Estanque doble fondo


Centre girder


Viga central


Solid floor


Piso sólido


Bottom longitudinals


Longitudinales del fondo


Transverse web


Nervio transversal


Side longitudinals


Longitudinales laterales


Frames


Cuadernas


Lower stool


Repisa inferior


Handhole covers


Tapas de registro


Bilge


Sentina


Ballast


Lastre


Tank top plating (Inner bottom)


Fondo o piso de la bodega


Bulkhead plain


or corrugated


Mamparo plano 


o coarrugado


Cross deck beams


Vigas transversales


Hatch coaming


Brazolas


Tank top longitudinals


Longitudinales del techo 


del estanque


Deck longitudinals


Longitudinales de cubierta


Hatch coaming


Inner part


Parte inferior


de la Brazola


Hatch cover


Tapa escotilla


Weather Deck


Cubierta principal


JDA




Saddle or Top side tank

Estanques 

laterales superiores

Shell

Planchaje 

Hopper tank

Estanques embudo 

Side girders

Vigas laterales 

Double bottom tank

Estanque doble fondo

Centre girder

Viga central

Solid floor

Piso sólido

Bottom longitudinals

Longitudinales del fondo

Transverse web

Nervio transversal

Side longitudinals

Longitudinales laterales

Frames

Cuadernas

Lower stool

Repisa inferior

Handhole covers

Tapas de registro

Bilge

Sentina

Ballast

Lastre

Tank top plating (Inner bottom)

Fondo o piso de la bodega

Bulkhead plain

or corrugated

Mamparo plano 

o coarrugado

Cross deck beams

Vigas transversales

Hatch coaming

Brazolas

Tank top longitudinals

Longitudinales del techo 

del estanque

Deck longitudinals

Longitudinales de cubierta

Hatch coaming

Inner part

Parte inferior

de la Brazola

Hatch cover

Tapa escotilla

Weather Deck

Cubierta principal

JDA





	HATCH & HOLD

NR 1
	G
	F
	P
	U
	Obs Nr

	Hatch covers
	
[image: image62.wmf]
	
	
	
	
	

	Hatch coamings
	
[image: image63.wmf]
	
	
	
	
	

	Rubber packing

Draining holes
	
[image: image64.wmf]
	
	
	
	
	

	Acting cleats

Compression bars
	
[image: image65.wmf]
	
	
	
	
	

	Non return valves
	
[image: image66.wmf]
	
	
	
	
	

	Hydraulic Systems 
	
[image: image67.wmf]
	
	
	
	
	

	Forward bulkhead
	
	
[image: image68.wmf]
	
	
	2
	

	After bulkhead
	
	
[image: image69.wmf]
	
	
	3
	

	Side/shell plating (port)
	
	
[image: image70.wmf]
	
	
	4
	

	Side/Shell plating (stb)
	
	
[image: image71.wmf]
	
	
	5
	

	Frames, brackets and bracket connection
	
[image: image72.wmf]
	
	
	
	
	

	Tank top plating
	
[image: image73.wmf]
	
	
	
	
	

	Hopper tank plating
	
[image: image74.wmf]
	
	
	
	
	

	Under deck plating
	
[image: image75.wmf]
	
	
	
	
	

	Manholes and covers
	
[image: image76.wmf]
	
	
	
	
	

	Ventilation, void spaces
	
[image: image77.wmf]
	
	
	
	
	

	Bilges, suctions
	
[image: image78.wmf]
	
	
	
	
	

	Ladders
	
	
[image: image79.wmf]
	
	
	6
	

	Air vents

Sounding pipes
	
[image: image80.wmf]
	
	
	
	
	

	Cleaning Condition
	
[image: image81.wmf]
	
	
	
	
	


OBSERVATIONS: 
Nr. 2:  Bulkhead Fwd, with paint stained and scratched (See photo: 28)

Nr. 3:  Bulkhead Aft, with paint stained and scratched (See photo: 29)
Nr. 4:  Ps plating, with paint stained and scratched (See photo: 31)

Nr. 5:  Ss plating, with paint stained and scratched (See photo: 32)

Nr. 6:  Bulkhead Aft, bent step ladder (See photo: 30)
	[image: image82.jpg]




	28.- Hold Nr. 1, Bulkhead Fwd, with paint stained and scratched.

	[image: image83.jpg]




	29.- Hold Nr. 1, Bulkhead Aft, with paint stained and scratched.

	[image: image260.png]


[image: image84.jpg]




	30.- Hold Nr. 1, Bulkhead Aft, bent step ladder.

	[image: image85.jpg]




	31.- Hold Nr. 1, Ps plating, with paint stained and scratched.

	[image: image86.jpg]




	32.- Hold Nr. 1, Ss plating, with paint stained and scratched.

	[image: image87.jpg]




	33.- Hold Nr. 1, Tank top plating, with normal sign of use. Scratched and rusty.

	[image: image88.jpg]




	34.- Hold Nr. 1, Manhole access in good condition.

	[image: image89.jpg]




	35.- Hold Nr. 1, Ps hatch coaming.

	[image: image90.jpg]




	36.- Hatch Nr.1, Compression bar in good condition.

	[image: image91.jpg]




	37.- Hatch Nr. 1, Hatch cover in good condition.

	[image: image92.jpg]




	38.- Hatch Nr. 1, Hydraulic piston in good conditions, operative.

	[image: image93.jpg]




	39.- Hatch Nr. 1, Acting cleat in good condition.


	HATCH & HOLD

NR 2
	G
	F
	P
	U
	Obs Nr

	Hatch covers
	
[image: image94.wmf]
	
	
	
	
	

	Hatch coamings
	
[image: image95.wmf]
	
	
	
	
	

	Rubber packing

Draining holes
	
[image: image96.wmf]
	
	
	
	
	

	Acting cleats

Compression bars
	
[image: image97.wmf]
	
	
	
	
	

	Non return valves
	
[image: image98.wmf]
	
	
	
	
	

	Hydraulic Systems 
	
[image: image99.wmf]
	
	
	
	
	

	Forward bulkhead
	
	
[image: image100.wmf]
	
	
	7
	

	After bulkhead
	
	
[image: image101.wmf]
	
	
	8
	

	Side/shell plating (port)
	
	
[image: image102.wmf]
	
	
	9
	

	Side/Shell plating (stb)
	
	
[image: image103.wmf]
	
	
	10
	

	Frames, brackets and bracket connection
	
[image: image104.wmf]
	
	
	
	
	

	Tank top plating
	
[image: image105.wmf]
	
	
	
	
	

	Hopper tank plating
	
[image: image106.wmf]
	
	
	
	
	

	Under deck plating
	
[image: image107.wmf]
	
	
	
	
	

	Manholes and covers
	
[image: image108.wmf]
	
	
	
	
	

	Ventilation, void spaces
	
[image: image109.wmf]
	
	
	
	
	

	Bilges, suctions
	
[image: image110.wmf]
	
	
	
	
	

	Ladders
	
	
[image: image111.wmf]
	
	
	11
	

	Air vents

Sounding pipes
	
[image: image112.wmf]
	
	
	
	
	

	Cleaning Condition
	
[image: image113.wmf]
	
	
	
	
	


OBSERVATIONS: 
Nr.   7:  Bulkhead Fwd, with paint stained and scratched (See photo: 40)

Nr.   8:  Bulkhead Aft, with paint stained and scratched (See photo: 41)

Nr.   9:  Ps plating, with paint stained and scratched (See photo: 43)

Nr. 10:  Ss plating, with paint stained and scratched (See photo: 44 & 45)

Nr. 11:  Bulkhead Aft, bent steps ladder (See photo: 42)

	[image: image114.jpg]




	40.- Hold Nr. 2, Bulkhead Fwd, with paint stained and scratched.

	[image: image115.jpg]




	41.- Hold Nr. 2, Bulkhead Aft, with paint stained and scratched.

	[image: image116.jpg]




	42.- Hold Nr. 2, Bulkhead Aft, bent steps ladder.

	[image: image117.jpg]




	43.- Hold Nr. 2, Ps plating, with paint stained and scratched.

	[image: image118.jpg]




	44.- Hold Nr. 2, Ss plating, with paint stained and scratched.

	[image: image119.jpg]




	45.- Hold Nr. 2, Ss plating, scratched and stained paint.

	[image: image120.jpg]




	46.- Hold Nr. 2, Tank top plating, with normal sign of use. Scratched and rusty.

	[image: image121.jpg]




	47.- Hold Nr. 2, Manhole access in good condition.

	[image: image122.jpg]




	48.- Hold Nr. 2, Ss hatch coaming.

	[image: image123.jpg]




	49.- Hatch Nr. 2, Compression bar in good condition.

	[image: image124.jpg]




	50.- Hatch Nr. 2, Hatch cover in good condition.

	[image: image125.jpg]




	51.- Hatch Nr. 2, Hydraulic piston in good conditions, operative.


	HATCH & HOLD

NR 3
	G
	F
	P
	U
	Obs Nr

	Hatch covers
	
[image: image126.wmf]
	
	
	
	
	

	Hatch coamings
	
[image: image127.wmf]
	
	
	
	
	

	Rubber packing

Draining holes
	
[image: image128.wmf]
	
	
	
	
	

	Acting cleats

Compression bars
	
[image: image129.wmf]
	
	
	
	
	

	Non return valves
	
[image: image130.wmf]
	
	
	
	
	

	Hydraulic Systems 
	
[image: image131.wmf]
	
	
	
	
	

	Forward bulkhead
	
	
[image: image132.wmf]
	
	
	12
	

	After bulkhead
	
	
[image: image133.wmf]
	
	
	13
	

	Side/shell plating (port)
	
	
[image: image134.wmf]
	
	
	14
	

	Side/Shell plating (stb)
	
	
[image: image135.wmf]
	
	
	15
	

	Frames, brackets and bracket connection
	
[image: image136.wmf]
	
	
	
	
	

	Tank top plating
	
[image: image137.wmf]
	
	
	
	
	

	Hopper tank plating
	
[image: image138.wmf]
	
	
	
	
	

	Under deck plating
	
[image: image139.wmf]
	
	
	
	
	

	Manholes and covers
	
[image: image140.wmf]
	
	
	
	
	

	Ventilation, void spaces
	
[image: image141.wmf]
	
	
	
	
	

	Bilges, suctions
	
[image: image142.wmf]
	
	
	
	
	

	Ladders
	
[image: image143.wmf]
	
	
	
	
	

	Air vents

Sounding pipes
	
[image: image144.wmf]
	
	
	
	
	

	Cleaning Condition
	
[image: image145.wmf]
	
	
	
	
	


OBSERVATIONS: 
Nr. 12:  Bulkhead Fwd, with paint stained and scratched (See photo: 52)

Nr. 13:  Bulkhead Aft, with paint stained and scratched (See photo: 53 & 54)

Nr. 14:  Ps plating, with paint stained and scratched (See photo: 55)

Nr. 15:  Ss plating, with paint stained and scratched (See photo: 56)

	[image: image146.jpg]




	52.- Hold Nr. 3, Bulkhead Fwd, with paint stained and scratched.

	[image: image147.jpg]




	53.- Hold Nr. 3, Bulkhead Aft, with paint stained and scratched.

	[image: image148.jpg]




	54.- Hold Nr. 3, Bulkhead Aft, scratched and stained paint.

	[image: image149.jpg]




	55.- Hold Nr. 3, Ps plating, with paint stained and scratched.

	[image: image150.jpg]




	56.- Hold Nr. 3, Ss plating, with paint stained and scratched.

	[image: image151.jpg]




	57.- Hold Nr. 3, view of bilge in good condition.

	[image: image152.jpg]




	58.- Hold Nr. 3, Tank top plating, with normal sign of use. Scratched and rusty.

	[image: image153.jpg]




	59.- Hold Nr. 3, Aft Australian ladder in good condition.

	[image: image154.jpg]




	60.- Hold Nr. 3, Ps hatch coaming.

	[image: image155.jpg]




	61.- Hatch Nr. 3, Compression bar in good condition.

	[image: image156.jpg]




	62.- Hatch Nr. 3, Hatch cover rubber packing in good condition.

	[image: image157.jpg]




	63.- Hatch Nr. 3, Hydraulic piston in good conditions, operative.


	HATCH & HOLD

NR 4
	G
	F
	P
	U
	Obs Nr

	Hatch covers
	
[image: image158.wmf]
	
	
	
	
	

	Hatch coamings
	
[image: image159.wmf]
	
	
	
	
	

	Rubber packing

Draining holes
	
[image: image160.wmf]
	
	
	
	
	

	Acting cleats

Compression bars
	
[image: image161.wmf]
	
	
	
	
	

	Non return valves
	
[image: image162.wmf]
	
	
	
	
	

	Hydraulic Systems 
	
[image: image163.wmf]
	
	
	
	
	

	Forward bulkhead
	
	
[image: image164.wmf]
	
	
	16
	

	After bulkhead
	
	
[image: image165.wmf]
	
	
	17
	

	Side/shell plating (port)
	
	
[image: image166.wmf]
	
	
	18
	

	Side/Shell plating (stb)
	
	
[image: image167.wmf]
	
	
	19
	

	Frames, brackets and bracket connection
	
[image: image168.wmf]
	
	
	
	
	

	Tank top plating
	
[image: image169.wmf]
	
	
	
	
	

	Hopper tank plating
	
[image: image170.wmf]
	
	
	
	
	

	Under deck plating
	
[image: image171.wmf]
	
	
	
	
	

	Manholes and covers
	
[image: image172.wmf]
	
	
	
	
	

	Ventilation, void spaces
	
[image: image173.wmf]
	
	
	
	
	

	Bilges, suctions
	
[image: image174.wmf]
	
	
	
	
	

	Ladders
	
[image: image175.wmf]
	
	
	
	
	

	Air vents

Sounding pipes
	
[image: image176.wmf]
	
	
	
	
	

	Cleaning Condition
	
[image: image177.wmf]
	
	
	
	
	


OBSERVATIONS: 
Nr. 16:  Bulkhead Fwd, with paint stained and scratched (See photo: 64)
Nr. 17:  Bulkhead Aft, with paint stained and scratched (See photo: 65)

Nr. 18:  Ps plating, with paint stained and scratched (See photo: 66)

Nr. 19:  Ss plating, with paint stained and scratched (See photo: 67 & 68)

	[image: image178.jpg]




	64.- Hold Nr. 4, Bulkhead Fwd, with paint stained and scratched.

	[image: image179.jpg]




	65.- Hold Nr. 4, Bulkhead Aft, with paint stained and scratched.

	[image: image180.jpg]




	66.- Hold Nr. 4, Ps plating, with paint stained and scratched.

	[image: image181.jpg]




	67.- Hold Nr. 4, Ss plating, with paint stained and scratched.

	[image: image182.jpg]




	68.- Hold Nr. 4, Ss plating, scratched and stained paint.

	[image: image183.jpg]




	69.- Hold Nr. 4, Tank top plating, with normal sign of use. Scratched and rusty.

	[image: image184.jpg]




	70.- Hold Nr. 4, Manhole access in good condition.

	[image: image185.jpg]




	71.- Hold Nr. 4, Ss hatch coaming. 

	[image: image186.jpg]




	72.- Hatch Nr. 4, Compression bar in good condition.

	[image: image187.jpg]




	73.- Hatch Nr. 4, Hatch cover in good condition.

	[image: image188.jpg]




	74.- Hatch Nr. 4, Hydraulic piston in good conditions, operative.

	[image: image189.jpg]




	75.- Hatch Nr. 4, Acting cleat in good condition.


	HATCH & HOLD

NR 5
	G
	F
	P
	U
	Obs Nr

	Hatch covers
	
[image: image190.wmf]
	
	
	
	
	

	Hatch coamings
	
[image: image191.wmf]
	
	
	
	
	

	Rubber packing

Draining holes
	
[image: image192.wmf]
	
	
	
	
	

	Acting cleats

Compression bars
	
[image: image193.wmf]
	
	
	
	
	

	Non return valves
	
[image: image194.wmf]
	
	
	
	
	

	Hydraulic Systems 
	
[image: image195.wmf]
	
	
	
	
	

	Forward bulkhead
	
	
[image: image196.wmf]
	
	
	20
	

	After bulkhead
	
	
[image: image197.wmf]
	
	
	21
	

	Side/shell plating (port)
	
	
[image: image198.wmf]
	
	
	22
	

	Side/Shell plating (stb)
	
	
[image: image199.wmf]
	
	
	23
	

	Frames, brackets and bracket connection
	
[image: image200.wmf]
	
	
	
	
	

	Tank top plating
	
[image: image201.wmf]
	
	
	
	
	

	Hopper tank plating
	
[image: image202.wmf]
	
	
	
	
	

	Under deck plating
	
[image: image203.wmf]
	
	
	
	
	

	Manholes and covers
	
[image: image204.wmf]
	
	
	
	
	

	Ventilation, void spaces
	
[image: image205.wmf]
	
	
	
	
	

	Bilges, suctions
	
[image: image206.wmf]
	
	
	
	
	

	Ladders
	
[image: image207.wmf]
	
	
	
	
	

	Air vents

Sounding pipes
	
[image: image208.wmf]
	
	
	
	
	

	Cleaning Condition
	
[image: image209.wmf]
	
	
	
	
	


OBSERVATIONS: 
Nr. 20:  Bulkhead Fwd, with paint stained and scratched (See photo: 76)

Nr. 21:  Bulkhead Aft, with paint stained and scratched (See photo: 77 & 78)

Nr. 22:  Ps plating, with paint stained and scratched (See photo: 79)

Nr. 23:  Ss plating, with paint stained and scratched (See photo: 80)

	[image: image210.jpg]




	76.- Hold Nr. 5, Bulkhead Fwd, with paint stained and scratched.

	[image: image211.jpg]




	77.- Hold Nr. 5, Bulkhead Aft, with paint stained and scratched.

	[image: image212.jpg]




	78.- Hold Nr. 5, Bulkhead Aft, scratched and stained paint.

	[image: image213.jpg]




	79.- Hold Nr. 5, Ps plating, with paint stained and scratched.

	[image: image214.jpg]] ;“ || | m
\ \ \‘A“ \
/ 4*\ \\t\\ \ \\
[ AN
il AR
i/ / . e b 2 sctaun 2 \R %
o4 S Y ’ #_ s :‘ B ‘1?,, vt g o ! \ \ P 3
.r:‘", ‘ / SR Zjl " ‘.’-If‘. ; “. :






	80.- Hold Nr. 5, Ss plating, with paint stained and scratched.

	[image: image215.jpg]




	81.- Hold Nr. 5, Tank top plating, with normal sign of use. Scratched and rusty.

	[image: image216.jpg]




	82.- Hold Nr. 5, Fwd Australian ladder in good condition.

	[image: image217.jpg]




	83.- Hold Nr. 5, Stevedore’s access rubber packing in good condition.

	[image: image218.jpg]




	84.- Hold Nr. 5, Ps hatch coaming.

	[image: image219.jpg]




	85.- Hatch Nr. 5, Compression bar in good condition.

	[image: image220.jpg]




	86.- Hatch Nr. 5, Hatch cover rubber packing in good condition.

	[image: image221.jpg]




	87.- Hatch Nr. 5, Hydraulic piston in good conditions, operative.


IX.   CARGO GEAR:
	[image: image222.emf]Ladder


Escala


Control Cabin


Caseta de control


 Wires


Cables


 Arm


Brazo


Pulley


Polea


Yard Arm


Penol


Engine Room 


Access


Acceso Sala 


de Máquina


 Hand Rail


Pasamanos


  Hook


Gancho


Glass


Vidrio


Runner: 


Virador


Guy: 


Amantillo


Prevent Guy: 


Contra amantillo 




Ladder

Escala

Control Cabin

Caseta de control

 Wires

Cables

 Arm

Brazo

Pulley

Polea

Yard Arm

Penol

Engine Room 

Access

Acceso Sala 

de Máquina

 Hand Rail

Pasamanos

  Hook

Gancho

Glass

Vidrio

Runner: Virador

Guy: Amantillo

Prevent Guy: Contra amantillo 




	Crane/DERRICK
Nr. 1
	G
	F
	P
	U
	Obs Nr
	CRANE/DERRICK
NR. 2
	G
	F
	P
	U
	Obs Nr

	Arm/Boom
	
[image: image223.wmf]
	
	
	
	
	
	Arm/Boom
	
[image: image224.wmf]
	
	
	
	
	

	Wires
	
[image: image225.wmf]
	
	
	
	
	
	Wires 
	
[image: image226.wmf]
	
	
	
	
	

	Winches
	
[image: image227.wmf]
	
	
	
	
	
	Winches
	
[image: image228.wmf]
	
	
	
	
	

	Controls  
	
[image: image229.wmf]
	
	
	
	
	
	Controls  
	
[image: image230.wmf]
	
	
	
	
	

	Railings & Ladders  
	
[image: image231.wmf]
	
	
	
	
	
	Railings & Ladders  
	
[image: image232.wmf]
	
	
	
	
	

	Hydraulic pipes
	
[image: image233.wmf]
	
	
	
	
	
	Hydraulic pipes
	
[image: image234.wmf]
	
	
	
	
	

	Control Cabin Glass 
	
[image: image235.wmf]
	
	
	
	
	
	Control Cabin Glass 
	
[image: image236.wmf]
	
	
	
	
	


	Crane/DERRICK

Nr. 3
	G
	F
	P
	U
	Obs Nr
	CRANE/DERRICK

NR. 4
	G
	F
	P
	U
	Obs Nr

	Arm/Boom
	
[image: image237.wmf]
	
	
	
	
	
	Arm/Boom
	
[image: image238.wmf]
	
	
	
	
	

	Wires
	
[image: image239.wmf]
	
	
	
	
	
	Wires 
	
[image: image240.wmf]
	
	
	
	
	

	Winches
	
[image: image241.wmf]
	
	
	
	
	
	Winches
	
[image: image242.wmf]
	
	
	
	
	

	Controls  
	
[image: image243.wmf]
	
	
	
	
	
	Controls  
	
[image: image244.wmf]
	
	
	
	
	

	Railings & Ladders  
	
[image: image245.wmf]
	
	
	
	
	
	Railings & Ladders  
	
[image: image246.wmf]
	
	
	
	
	

	Hydraulic pipes
	
[image: image247.wmf]
	
	
	
	
	
	Hydraulic pipes
	
[image: image248.wmf]
	
	
	
	
	

	Control Cabin Glass 
	
[image: image249.wmf]
	
	
	
	
	
	Control Cabin Glass 
	
[image: image250.wmf]
	
	
	
	
	


OBSERVATIONS: The cranes were observed from deck area, apparently they are operatives and without damage.
	[image: image251.jpg]




	88.- View of Crane Nr. 1.

	[image: image252.jpg]




	89.- Crane Nr. 1, view of external cabin condition.

	[image: image253.jpg]




	90.- View of Crane Nr. 2.

	[image: image254.jpg]




	91.- View of Crane Nr. 2, SWL 30.5 Tons.


	[image: image255.jpg]




	92.- View of Crane Nr. 3.

	[image: image256.jpg]




	93.- View of Crane Nr. 4.


	[image: image257.jpg]




	94.- Crane Nr. 4, view of winch and wires condition.

	[image: image258.jpg]




	95.- Crane Nr. 4, view of inner cabin condition.


X. BUNKER SURVEY:
The following quantities were found onboard as calculated by Chief Engineer and the undersigned surveyor, from the ship’s calibration curves with due considerations to the trim of the vessel, temperature and specific gravity of oil:
	Draft Fwd
	5.10 m
	Sea water temp.
	17° C

	Draft Aft
	6.89 m
	Engine Room temp.
	27° C

	Trim
	1.90 m
	
	                    


Bunker survey was carried out at Chancay port, Pier 2-B of COSCO Shipping Ports Chancay Peru S.A., on January 11th, 2026, from 16:00 Hrs to 17:00 Hrs. LT, and quantities found are the following:
FUEL OIL (VERY LOW SULPHUR)
	Tank
	Sounding/

Ullage
	Volume
	Density
	Temp.
	V.C.F.
	G.S.V. m3
	Metric

	
	Corr’d (m)
	CB
	@15° C
	Deg (C)
	T (54B)
	@15° C
	Tons

	Nº 1 H.F.O. “P” 
	(U) 1.142
	268.760
	0.9469
	25.0
	0.9928
	266.825
	252.363

	Nº 1 H.F.O. “S”
	(U) 0.909
	302.740
	0.9400
	18.0
	0.9978
	302.074
	283.617

	Nº 2 H.F.O. “P”
	(U) 0.910
	303.340
	0.9400
	18.0
	0.9978
	302.673
	284.180

	Nº 2 H.F.O. “S”
	0.000
	1.830
	0.9469
	18.0
	0.9978
	1.826
	1.727

	Nº 3 H.F.O. “P”
	0.212
	0.342
	0.9580
	18.0
	0.9979
	0.341
	0.326

	Nº 3 H.F.O. “S”
	0.560
	1.210
	0.9500
	18.0
	0.9978
	1.207
	1.145

	H.F.O. SETT. TK.
	GAUGED
	7.800
	0.9469
	60.0
	0.9672
	7.544
	7.135

	H.F.O. SERV. TK.
	GAUGED
	18.300
	0.9469
	95.0
	0.9413
	17.226
	16.292

	LSFO SETT. TK
	GAUGED
	3.100
	0.9580
	69.0
	0.9612
	2.980
	2.852

	LSFO SERV. TK.
	GAUGED
	3.250
	0.9580
	56.0
	0.9706
	3.154
	3.018

	OVERFLOW
	GAUGED
	0.200
	0.9469
	20.0
	0.9964
	0.199
	0.188

	                                                                          TOTAL Cubic Metres @ 15° C
	906.049
	852.843

	TOTAL Metric Tons
	 
	


MARINE GAS OIL (LOW SULPHUR)
	Tank
	Sounding/

Ullage
	Volume
	Density
	Temp.
	V.C.F.
	G.S.V. m3
	Metric

	
	Corr’d (m)
	CB
	@15° C
	Deg (C)
	T (54B)
	@15° C
	Tons

	Nº 1 D.O. “S”
	2.560
	18.120
	0.8761
	18.0
	0.9976
	18.077
	15.817

	Nº 2 D.O. “P”
	1.740
	10.970
	0.8761
	18.0
	0.9976
	10.944
	9.576

	Nº 3 D.O. “P”
	3.121
	39.391
	0.8600
	18.0
	0.9975
	39.293
	33.749

	N° 1 D.O. SERV. TK
	GAUGED
	4.600
	0.8600
	32.0
	0.9860
	4.536
	3.896

	N° 2 D.O. SERV. TK
	GAUGED
	6.740
	0.8600
	32.0
	0.9860
	6.646
	5.708

	                                                                          TOTAL Cubic Metres @ 15° C
	79.496
	68.746

	TOTAL Metric Tons
	 
	


Quantities of Bunkers found at Bunker Survey time, on January 11th, 2026, at 17:00 LT hours (January 11th, 2026, at 22:00 hours UTC):
	Fuel Oil (Very Low Sulphur)
	  852.843 MT

	Marine Gas Oil (Low Sulphur)
	    68.746 MT


Consumption of Bunkers between Bunker survey and Re-delivery time:
	Fuel Oil (Very Low Sulphur)
	     2.120 MT

	Marine Gas Oil (Low Sulphur)
	          0.000 MT


Quantity of Bunker at Re-delivery time at DLOSP, in the port of Chancay, Peru, on January 12th, 2026, at 07:30 hours LT (January 12th, 2026, at 12:30 hours UTC).
	Fuel Oil (Very Low Sulphur)
	   850.723 MT

	Marine Gas Oil (Low Sulphur)
	     68.746 MT


XI.        ATTENDING PARTIES:
	Name
	Position
	Company
	On behalf of

	Mr. Jose Mari L. Cabanatan
	Captain
	MV. Leo Ocean
	Owners

	Mr. Marlon C. Jalandoni
	Chief Officer
	MV. Leo Ocean
	Owners

	Mr. Jess M. Amparo
	Chief Engineer
	MV. Leo Ocean
	Owners

	Mr. Felipe Guillén C.
	Marine surveyor
	ALS Inspection
	Dampskibsselskabet Norden

	Mr. Roberto Guillén B.
	Marine surveyor
	ALS Inspection
	Dampskibsselskabet Norden


XII.        ATTACHMENTS:
· Re-delivery Certificate
· Bunker Survey Certificate

· Delivery Statement (ship)
This survey was carried out without prejudice and in the interest of whom it may concern.

Eng. Felipe Guillén Curo

ALS Surveyor

Revised by:    R. Guillén 
Approved by: J. Alvarez
PAGE  
	
	P1 of 63

	Right Solutions • Right Partner
	www.alsglobal.com    



_1139343182.bin

