	
	ALS Inspection Chile SpA 
Limache 3405, Office 61
Viña del Mar, CHILE

T +56 32 2545 500

Report: ANF-2605-1807
Date: May 17, 2026


[image: image4.jpg]|
|
|
/I\\ [ tmars]
| <
i
,

(|, e
== b : /
» PR

- ° 3 ,,,_. Av_f.“s’/w
= A." \ ““% hmn
S . (R
= H A
PN
— . Ou

S

e B )





This is to certify that we, ALS INSPECTION CHILE SPA, at the request of DAMPSKIBSSELSKABET NORDEN A/S. and under their master’s consent, the undersigned surveyor attended onboard:
MV “AZTECA” 
[image: image5.jpg]AZTECA

MAJURO
IMO S646742




[image: image6.png]right solutions.
right partner.




On May 16, 2026, while vessels were at anchorage in Mejillones Bay / CHILE, with the purpose of carry-out a:

“BUNKERING SURVEY”
And does report as follows:

1. SCOPE OF SURVEY

The present survey was carried out according to instructions received from Mr. Daniel Duque Martins from DAMPSKIBSSELSKABET NORDEN A/S., in order to ascertain bunker quantities of Very Low Sulphur Fuel Oil received by MV “AZTECA” from MT “DON PANCHO II”, as per measurements performed onboard the receiver vessel.
2. VESSEL’S PARTICULARS
	Vessel
	MT “Don Pancho II”  

(Supplying barge)
	MV “AZTECA”
 (Receiver vessel)

	Flag
	Chilean
	Marshall Island

	Port of Registry
	Valparaíso
	Majuro

	Owners
	Compañia Maritima Chilena 
	Ariston Navigation Corp

	Call sign
	CBDH
	3FJWS

	G.R.T.
	3,000 GRT
	22,716.00 GRT

	Chief Officer
	Mr. Christian Mesenet 
	Mr. Kynnan Abella

	Chief Engineer
	Mr. ---
	Mr. Jason Gammad

	Master
	Capt. Cristian Jimenez
	Capt. Ernesto Lo


3. BUNKER SURVEY REPORT

Measurements and calculations onboard MV “AZTECA” were carried-out jointly with vessel’s Chief Engineer, Mr. Jason Gammad and Fourth Engineer, Mr. Michael Jayoma .
Measurements and calculations onboard MT “Don Pancho” were carried out by barge’s Chief Officer, Mr. Messenet.

Measurements and calculations were carried out with due consideration to vessel’s calibration curves, trim of vessels, temperature, and specific gravity of oil.

All tanks were sounded upon initial and final bunker surveys.

3.1. 
Date / Times:
Onboard MV “Azteca”:
Bunker survey commenced
: May 16, 2022 - 07:45 hours LT / 11:45 hours UTC
Bunker survey completed
: May 16, 2022 - 14:15 hours LT / 18:15 hours UTC
3.2.  
Initial bunker survey on M/V “AZTECA”: 

[image: image1.emf]Sea water

Engine room

To be corrected

VSLFO LSMGO Date

14.25 0 Place

2.80 0 VSLFO LSMGO

0.70 0 0.956 0.8530

Max

SoundingSounding

VolumeTemp.

Sp.gravityCorrection

Correction WEIGHT

Capacity cm

corrected

(cbm) ºC / ºF 15ºC T.54B T.56 Metric tons

225.61 7.09 7.09 118.460 28.0 0.9560 0.9907 0.9549 112.066

271.36 0.00 0.00 0.000 19.0 0.9560 0.9971 0.9549 0.000

HO 2P 420.41 0.00 0.00 0.000 19.0 0.9512 0.9971 0.9501 0.000

HO LS FO 2S 451.01 0.00 0.00 0.000 19.0 0.9589 0.9972 0.9578 0.000

LSFO SERV 1 37.90 Gauge Gauge 33.140 98.0 0.9560 0.9399 0.9549 29.743

HL LS FO SER 2 37.90 Gauge Gauge 35.100 85.0 0.9560 0.9494 0.9549 31.821

LS FO SETT 1 37.90 Gauge Gauge 17.150 84.0 0.9560 0.9501 0.9549 15.560

HO LS FO SETT 2 37.90 Gauge Gauge 28.100 65.0 0.9560 0.9640 0.9549 25.866

HO OVER 20.76 0.08 0.08 0.220 25.0 0.9512 0.9928 0.9501 0.208

TOTAL : 215.264

Max

SoundingSounding

VolumeTemp.

Sp.gravityCorrection

Correction WEIGHT

Capacity

corrected

(cbm) ºC / ºF 15ºC T.54B T.56 Metric tons

65.50 0.04 0.04 1.320 20.0 0.8729 0.9960 0.8718 1.146

169.63 1.98 1.98 77.800 20.0 0.8530 0.9959 0.8519 66.006

86.88 0.66 0.66 14.500 20.0 0.8715 0.9960 0.8704 12.570

MGO SERV 1 17.30 Gauge Gauge 9.100 32.7 0.8715 0.9857 0.8704 7.808

MGO SERV 2 17.30 Gauge Gauge 8.400 29.0 0.8715 0.9887 0.8704 7.229

DO SERV 17.30 Gauge Gauge 10.615 34.0 0.8715 0.9847 0.8704 9.098

26.98 0.52 0.52 4.440 25.0 0.8715 0.9919 0.8704 3.833

TOTAL : 107.690

HO DO OVER

MGO P  MDO P

MGOS MDOS S

DOS

Tank



Average consumption per day

15.0

Aft

Forward



Tank



At port

VERY LOW SULPHUR FUEL OIL

Maneuvering

Specific 

gravity



LSFO

LOW SULPHUR MARINE GAS OIL 

Cristobal, Panama

April 17th, 2026



Last bunkering information

Sailing

HFO 1P

0.50

TEMPERATURES (ºC)

14.0

26.0

Trim

DRAFTS (m)

9.32

9.82


3.3. 
Final bunker survey on M/V “AZTECA”: 

[image: image2.emf]Sea water

Engine room

To be corrected

Max

SoundingSounding

VolumeTemp.

Sp.gravityCorrection

Correction WEIGHT

Capacity cm

corrected

(cbm) ºC / ºF 15ºC T.54B T.56 Metric tons

225.61 7.09 7.09 118.46 28.0 0.9560 0.9907 0.9549 112.066

LSFO 271.36 0.00 0.00 0.00 19.0 0.9560 0.9971 0.9549 0.000

HO 2P 420.41 6.45 6.45 192.10 35.0 0.9498 0.9856 0.9487 179.615

HO LS FO 2S 451.01 3.60 3.60 322.32 40.0 0.9498 0.9819 0.9487 300.261

LSFO SERV 1 37.90 Gauge Gauge 33.14 98.0 0.9560 0.9399 0.9549 29.743

HL LS FO SER 2 37.90 Gauge Gauge 35.10 85.0 0.9560 0.9494 0.9549 31.821

LS FO SETT 1 37.90 Gauge Gauge 17.15 84.0 0.9560 0.9501 0.9549 15.560

HO LS FO SETT 2 37.90 Gauge Gauge 28.10 65.0 0.9560 0.9640 0.9549 25.866

HO OVER 20.76 0.08 0.08 0.22 25.0 0.9512 0.9928 0.9501 0.208

TOTAL : 695.140

Max

SoundingSounding

VolumeTemp.

Sp.gravityCorrection

Correction WEIGHT

Capacity

corrected

(cbm) ºC / ºF 15ºC T.54B T.56 Metric tons

65.50 19.00 19.00 1.320 20.0 0.8729 0.9960 0.8718 1.146

169.63 1.62 1.62 77.800 20.0 0.8530 0.9959 0.8519 66.006

86.88 1.58 1.58 14.500 20.0 0.8715 0.9960 0.8704 12.570

17.30 Gauge Gauge 9.100 32.7 0.8715 0.9857 0.8704 7.808

17.30 Gauge Gauge 8.400 29.0 0.8715 0.9887 0.8704 7.229

17.30 Gauge Gauge 10.615 34.0 0.8715 0.9847 0.8704 9.098

26.98 0.52 0.52 4.440 25.0 0.8715 0.9919 0.8704 3.833

TOTAL : 107.690

HO DO OVER

MGO SERV 1

DRAFTS (m) TEMPERATURES (ºC)

Trim

Forward 9.11 14.0

Aft 10.11 29.0

1.00 15.0



VERY LOW SULPHUR FUEL OIL

Tank



HFO 1P



DOS

MGO P  MDO P

MGOS MDOS S

MGO SERV 2

DO SERV

LOW SULPHUR MARINE GAS OIL 

Tank


3.4. bunker survey on M/T “Don Pancho II”:
[image: image3.emf]
3.5. Differences between initial and final bunker surveys:

Onboard M/V “AZTECA:
	INITIAL BUNKER SURVEY

Metric Tons
	INITIAL B. S.

Metric Tons
	FINAL B. S.

Metric Tons
	DIFFERENCE

Metric Tons

	VERY LOW SULPHUR FUEL OIL (VLSFO)
	215.264
	695.140
	479.876

	LSMGO
	107.690
	107.690
	0.00


From M/T “Don Pancho”:
	INITIAL BUNKER SURVEY

Metric Tons
	INITIAL B. S.

Metric Tons
	FINAL B. S.

Metric Tons
	DIFFERENCE

Metric Tons

	VERY LOW SULPHUR FUEL OIL (VLSFO)
	2528.70
	2048.60
	480.10

	LSMGO
	594.20
	594.20
	0.00


4. FINAL FIGURES

TOTAL QUANTITIES RECEIVED BY M/V “AZTECA” (according to ALS Surveyor´s calculation):
	VERY LOW SULPHUR FUEL OIL
	479.876 Metric Tons

	MARINE GAS OIL LOW SULPHUR
	000.00 Metric Tons


TOTAL QUANTITIES DELIVERED BY M/T “DON PANCHO II” (according to Barge Chief Officer’s calculation):
	VERY LOW SULPHUR FUEL OIL
	480.100 Metric Tons

	MARINE GAS OIL LOW SULPHUR
	000.00 Metric Tons


TOTAL QUANTITIES DELIVERED BY M/T “DON PANCHO II” (according to official documents BDR):
	VERY LOW SULPHUR FUEL OIL
	480.00 Metric Tons

	MARINE GAS OIL LOW SULPHUR   
	000.00 Metric Tons


5. SAMPLE SEAL NUMBERS

	Vessel Sampling
	Barge Sample Delivery

	MARPOL
	008275
	Barge retention
	008271

	Vessel retention
	80529704
	MARPOL
	008275

	Supplier retention
	80529705
	Vessel retention
	008273

	Laboratory
	80529703
	Barge retention
	008272


6. GENERAL REMARKS
· MV “AZTECA” did not receive Diesel oil.  
· Sampling and bunkering processes were conducted according to Marpol 73/78 Annex Nr. 6, regulation Nr. 18 -6 (Fuel Oil Quality).
· Samples were delivered by Chief Officer of M/T “Don Pancho II” to Chief Engineer of MV “AZTECA”.
· The product delivered was sampled by dripping method on the respective manifold of each vessel during all bunkering process.
· All bottles, seals and containers were new.
7. ATTACHMENTS

· M/V “AZTECA” Bunker Certificate issued by ALS
· Bunker Delivery Receipt
This survey report was carried out without prejudice and in the interest of whom it may concern
ALS Inspection Chile Surveyor

Prepared by
: M. Lopez
Reviewed by
: A, Paredes
Approved by
: J. Lopez
	Right Solutions - Right Partner                                                                                                                                                             alsglobal.com



	Page 8 of 8
 



