MV “AQUAMARINE”

FULL ON HIRE & BUNKER SURVEY

This is to certify that the undersigned representative of ALS Inspection Chile did, at the request of Messrs. DAMPSKIBSSELSKABET NORDEN A/S attend on board motor vessel “AQUAMARINE SW” with Master consent on May 19th, 2026, while she was at anchorage in MEJILLONES BAY, Chile, in order to carry out an On Hire, general condition survey and bunker survey, as follows:

COMMENTS BY SURVEYOR

Vessel with certificates updated; hull was observed by Port side and starboard side and was observed with paint scratched, stained and with rust; hull dented port side after part superstructure in good condition, railing with several dented; cargo holds were observed clean and dry, with ladder and ladder step bent, shell plating with paint scratched; hatchcovers were in apparent good structural conditions, some hydraulic piston to opening the hatchcovers were noted with oil leakage; cargo cranes were observed in apparent good conditions.

INDEX

| Content |  | Page Nr. |
| --- | --- | --- |
| I. | Times |  |
| II. | Vessel’s particulars |  |
| III. | Capacities |  |
| IV. | Certificates Status |  |
| V. | External Hull Condition |  |
| VI. | General Statement of hull structure |  |
| VII. | Deck |  |
| VIII. | Holds & Hatches |  |
| IX. | Cargo Gear Conditions |  |
| X. | Bunker Survey |  |
| XI. | Attending Parties |  |
| XII. | Attachments |  |

# TERMS AND Definitions of Condition

| Good | G | Unimpaired condition without significant wear or deviation from original strength and operating efficiency. No maintenance or repair required. |
| --- | --- | --- |
| Fair | F | Condition in which wear and tear and other deficiencies of minor nature not requiring correction or repair. |
| Poor | P | Condition in which the adequacy of strength and/or operational efficiency is marginally below acceptable limits or is in doubt. Remedial action is required. |
| Unsatisfactory | U | Condition of undoubtedly inadequate strength, or operational efficiency. Immediate extensive repair/renewal, required to reinstate serviceability. |
| Observation | Obs Nr. | Indicate observation number (a special condition or important remark). |

SCOPE OF SURVEY

- To verify capacities and general arrangement plans, load manuals and all certificates and to report upon them.

- To examine the weather deck and superstructures, cargo spaces including the cargo cranes, cargo gear and to record all defects.

- To examine the main hull/structures and fittings in each cargo space and to record all defects.

- To examine vessel's condition, cleanliness, and load readiness and to record all defects.

- To establish by sounding and calculation the quantity of fuel on board at the time of survey and to establish the fuel consumption from the time the vessel became On-Hire to the time of sounding.

- TIMES

| Date | Time | Description |
| --- | --- | --- |
| May 19, 2026 | 09:30 | ALS Surveyor on board MV “AQUAMARINE SW” |
|  | 09:45 - 10:00 | Key meeting with C/E |
|  | 10:00 – 11:00 | Vessel inspection holds and structures |
|  | 11:00 | Bunker Survey commenced |
|  | 13:00 | Bunker Survey completed |
|  | 13:00 - 13:45 | Vessel certificates revison |
|  | 13:45 – 14:00 | Documents signed |
|  | 14:30 | ALS surveyor left the vessel |

- VESSEL’S PARTICULARS

| Master Name | Mr. Kang, Huaihong |
| --- | --- |
| Chief Officer Name | Mr. Ding , Jie |
| Chief Engineer | Mr. Li, Yongkang |
| Flag & Port of Register | Panama & Panama |
| Registered Owners | Brave Pescadores SA |
| Year Built | 2012 |
| Length Overall | 177.8 Mts |
| Breath | 169.8 Mts |
| Summer. Draft | 10.869 Mt |
| GRT | 22, 855 Mt |
| NRT | 12.519 Mt |
| Classification Society | N.K. |
| Cargo gear | 4 x 30 T x 26 Mts Elect-Hyu |

- CAPACITIES

| Cargo Capacity | 45,790.76 m3 Grain // 45,294.41 m3 Bale |
| --- | --- |
| Summer. DWT | 37,119. Mt |
| IFO / MGO | 1,543.18 m3 // 170.14 m3 |
| Lub Oil | 27.6 m3 |
| Hydraulic Oil | 20.2 m3 |
| Fresh Water | 219.28 m3 |
| Ballast Water | 14,305.40 m3 |

- CERTIFICATES STATUS

| Certificate / Document | Issue date | Expiry date | Endorsement |
| --- | --- | --- | --- |
|  | Issue place |  |  |
| Certificate of Registry | 12-07-2022 | 02-10-2027 |  |
|  | Panama, Panama |  |  |
| Certificate of Class | Shanghai, China | 05-07-2027 |  |
|  | 02-07-2022 |  |  |
| International Tonnage | 06-09-2012 |  |  |
|  | Usuki, Japan |  |  |
| Suez Canal Tonnage | 06-09-2012 |  |  |
|  | Usuki, Japan |  |  |
| Panama Tonnage | 06-09-2012 |  |  |
|  | Usuki, Japan |  |  |
| International load line | 02-07-2022 | 05-09-2027 |  |
|  | Shanghai, China |  |  |
| Cargo ship safety construction | 02-07-2022 | 05-09-2027 |  |
|  | Shanghai, China |  |  |
| Cargo Ship Safety Equipment | 04-10-2024 | 05-09-2027 |  |
|  | Tokyo, Japan |  |  |
| Cargo Ship Safety Radio | 04-10-2024 | 05-09-2027 |  |
|  | Tokyo, Japan |  |  |
| Annual Cargo Gear Examination | 09-01-26 |  |  |
|  | Singapore, Singapore |  |  |
| International Oil Pollution Prevention | 02-07-2022 | 05-09-2027 |  |
|  | Shanghai, China |  |  |
| International Air Pollution Prevention | 02-07-2022 | 05-09-2027 |  |
|  | Zhou Shan, China |  |  |
| International Ship Security | 14-11-2022 | 25-12-2027 |  |
|  | Panama, Panama |  |  |
| Last Port State Control | May 14, 20926 |  |  |
|  | Paracas, Pisco, Peru |  |  |
| Last dry dock | July 20th, 2025 |  |  |
|  | Zhou Shan, China |  |  |

# V. EXTERNAL HULL CONDITION

|  | G | F | P | U | Obs Nr |  |
| --- | --- | --- | --- | --- | --- | --- |
| Structural condition |  |  |  |  | 1 |  |
| Overall appearance |  |  |  |  |  |  |
| Coating condition |  |  |  |  |  |  |

OBSERVATIONS: Nr. 1:

Hull Ps/Ss with paint scratched, stained and rusty

# VI. GENERAL STATEMENT OF HULL STRUCTURE

| Upon viewing vessel from various angles, were there any apparent or obvious signs of major longitudinal or transverse deformation or structural failure which might indicate earlier serious damage or poor repairs? | Y | N |
| --- | --- | --- |

- General view of hull Fwd, paint scratched and rusty

- Hull Fwd/Ss, paint scratched and rusty

- Hull Ss, general conditions

- Hull Ss, paint scratched and rusty

- Hull Aft/Ss, paint scratched and rusty

- Hull Aft/Ss, paint scratched and rusty

- Hull After part, paint scratched and rusty

- Hull Aft/Ps, dented FW tank position

- Hull Aft/Ps, paint scratched and rusty

- Hull Mid part/Ps, paint scratched and rusty

- Hull Fwd/Ps, paint scratched and rusty

VII. DECK

| FORECASTLE DECK | G | F | P | U | Obs Nr |  | POOP DECK | G | F | P | U | Obs Nr |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor equipment |  |  |  |  |  |  | Winches |  |  |  |  |  |  |
| Railings and bulwark |  |  |  |  |  |  | Systems and bulkwark |  |  |  |  |  |  |
| Air and sounding pipes |  |  |  |  |  |  | Air and sounding pipes |  |  |  |  |  |  |
| Plating |  |  |  |  |  |  | Plating |  |  |  |  |  |  |
| Mooring lines |  |  |  |  |  |  | Mooring lines |  |  |  |  |  |  |
| Light mast |  |  |  |  |  |  | Access |  |  |  |  |  |  |
| Bollards |  |  |  |  |  |  | Bollards |  |  |  |  |  |  |
| Fair leaders |  |  |  |  |  |  | Fair leaders |  |  |  |  |  |  |

| PORT SIDE DECK | G | F | P | U | Obs Nr |  | STB SIDE DECK | G | F | P | U | Obs Nr |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hatchcoaming |  |  |  |  |  |  | Hatchcoaming |  |  |  |  |  |  |
| Deck plating |  |  |  |  |  |  | Deck plating |  |  |  |  |  |  |
| Railings / bulwark |  |  |  |  | 2 |  | Railings / bulwark |  |  |  |  |  |  |
| Piping in way |  |  |  |  |  |  | Piping in way |  |  |  |  |  |  |
| Air and sounding pipes |  |  |  |  |  |  | Air and sounding pipes |  |  |  |  |  |  |
| Stevedore’s access |  |  |  |  |  |  | Stevedore’s access |  |  |  |  |  |  |

Observations:

Nr. 2: Railing several dented alongside vessel, port and star board side

- Forward stations conditions pivot in good conditions

- Bitts and general conditions forward station

- Main view of railing and bitts conditions

- Another view of pivot and bulk wards conditions

- Main view of roller conditions

- Ropes conditions on forwards stations

- Main deck Ss railing dented

- Another view of main deck SS with railing dented

- Main deck SS condition and railing dented.

- Main deck Ss. railing several dented

- Main deck Ps. Railing dented

- Main deck Ps, general conditions

- Main deck Ps. Railing dented

- Main deck Ps general conditions

- After station in good conditions

- After station bitts and railing in good conditions

- Windlass in sound conditions

- View of railing rollers and bitts in good conditions

- Ropes conditions

- Anotther view of ropes in good conditions

# VIII. HOLDS & HATCHES

| HATCH & HOLD NR. 1 | G | F | P | U | Obs Nr |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hatch covers |  |  |  |  |  |  |
| Hatch coamings |  |  |  |  |  |  |
| Rubber packing Draining holes |  |  |  |  |  |  |
| Acting cleats Compression bars |  |  |  |  |  |  |
| Non return valves |  |  |  |  |  |  |
| Hydraulic Systems |  |  |  |  |  |  |
| Forward bulkhead |  |  |  |  | 4 |  |
| After bulkhead |  |  |  |  | 4 |  |
| Side/shell plating (port) |  |  |  |  | 4 |  |
| Side/shell plating (stb) |  |  |  |  | 4 |  |
| Frames, brackets and bracket connection |  |  |  |  |  |  |
| Tank top plating |  |  |  |  | 4 |  |
| Hopper tank plating |  |  |  |  | 4 |  |
| Under deck plating |  |  |  |  |  |  |
| Manholes and covers |  |  |  |  |  |  |
| Ventilation, void spaces |  |  |  |  |  |  |
| Bilges, suctions |  |  |  |  |  |  |
| Ladders |  |  |  |  |  |  |
| Air vents Sounding pipes |  |  |  |  |  |  |
| Cleaning Condition |  |  |  |  |  |  |

OBSERVATIONS:

Nr. 4: Bulkhead fwd, lower stool with paint scratched and rust.

- Hold Nr. 1, bulkhead Fwd.

- Hold Nr. 1, after bulkhead

- Hold Nr. 1, star board side plate scratches and rusted

- Hold Nr. 1, Port side shell with scratches

- Hold Nr. 1, hydraulic system in order

- Hold Nr. 1, rubber and compression bar in good conditions

- Hold Nr. 1, drain channel and compression bar in good conditions

- Hold Nr. 1, roller rusted in good conditions

| HATCH & HOLD NR. 2 | G | F | P | U | Obs Nr |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hatch covers |  |  |  |  |  |  |
| Hatch coamings |  |  |  |  |  |  |
| Rubber packing Draining holes |  |  |  |  |  |  |
| Acting cleats Compression bars |  |  |  |  |  |  |
| Non return valves |  |  |  |  |  |  |
| Hydraulic Systems |  |  |  |  |  |  |
| Forward bulkhead |  |  |  |  | 4 |  |
| After bulkhead |  |  |  |  | 4 |  |
| Side/shell plating (port) |  |  |  |  | 4 |  |
| Side/shell plating (stb) |  |  |  |  | 4 |  |
| Frames, brackets and bracket connection |  |  |  |  |  |  |
| Tank top plating |  |  |  |  | 4 |  |
| Hopper tank plating |  |  |  |  | 4 |  |
| Under deck plating |  |  |  |  |  |  |
| Manholes and covers |  |  |  |  |  |  |
| Ventilation, void spaces |  |  |  |  |  |  |
| Bilges, suctions |  |  |  |  |  |  |
| Ladders |  |  |  |  |  |  |
| Air vents Sounding pipes |  |  |  |  |  |  |
| Cleaning Condition |  |  |  |  |  |  |

OBSERVATIONS:

Nr. 4: Bulkhead fwd, lower stool with paint scratched and rust.

- Hold Nr. 2, bulkhead Fwd.

- Hold Nr. 2, after bulkhead

- Hold Nr. 2, star board side plate scratches and rusted

- Hold Nr. 2, Port side shell with scratches

- Hold Nr. 2, hydraulic system in order

- Hold Nr. 2, non-return valve in good conditions

- Hold Nr. 2, drain channel and compression bar in good conditions

- Hold Nr. 2, roller rusted in good conditions

| HATCH & HOLD NR. 3 | G | F | P | U | Obs Nr |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hatch covers |  |  |  |  |  |  |
| Hatch coamings |  |  |  |  |  |  |
| Rubber packing Draining holes |  |  |  |  |  |  |
| Acting cleats Compression bars |  |  |  |  |  |  |
| Non return valves |  |  |  |  |  |  |
| Hydraulic Systems |  |  |  |  |  |  |
| Forward bulkhead |  |  |  |  | 4 |  |
| After bulkhead |  |  |  |  | 4 |  |
| Side/shell plating (port) |  |  |  |  | 4 |  |
| Side/shell plating (stb) |  |  |  |  | 4 |  |
| Frames, brackets and bracket connection |  |  |  |  |  |  |
| Tank top plating |  |  |  |  | 4 |  |
| Hopper tank plating |  |  |  |  | 4 |  |
| Under deck plating |  |  |  |  |  |  |
| Manholes and covers |  |  |  |  |  |  |
| Ventilation, void spaces |  |  |  |  |  |  |
| Bilges, suctions |  |  |  |  |  |  |
| Ladders |  |  |  |  |  |  |
| Air vents Sounding pipes |  |  |  |  |  |  |
| Cleaning Condition |  |  |  |  |  |  |

OBSERVATIONS:

Nr. 4: Bulkhead fwd, lower stool with paint scratched and rust.

- Hold Nr. 3, bulkhead Fwd.

- Hold Nr. 3, after bulkhead

- Hold Nr. 3, star board side plate scratches and rusted

- Hold Nr. 3, Port side shell with scratches

- Hold Nr. 3, hydraulic system in order

- Hold Nr. 3, non-return valve in good conditions

- Hold Nr. 3, actean cleats rusted, operative

- Hold Nr. 3, roller rusted in good conditions

| HATCH & HOLD NR. 4 | G | F | P | U | Obs Nr |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hatch covers |  |  |  |  |  |  |
| Hatch coamings |  |  |  |  |  |  |
| Rubber packing Draining holes |  |  |  |  |  |  |
| Acting cleats Compression bars |  |  |  |  |  |  |
| Non return valves |  |  |  |  |  |  |
| Hydraulic Systems |  |  |  |  |  |  |
| Forward bulkhead |  |  |  |  | 4 |  |
| After bulkhead |  |  |  |  | 4 |  |
| Side/shell plating (port) |  |  |  |  | 4 |  |
| Side/shell plating (stb) |  |  |  |  | 4 |  |
| Frames, brackets and bracket connection |  |  |  |  |  |  |
| Tank top plating |  |  |  |  | 4 |  |
| Hopper tank plating |  |  |  |  | 4 |  |
| Under deck plating |  |  |  |  |  |  |
| Manholes and covers |  |  |  |  |  |  |
| Ventilation, void spaces |  |  |  |  |  |  |
| Bilges, suctions |  |  |  |  |  |  |
| Ladders |  |  |  |  |  |  |
| Air vents Sounding pipes |  |  |  |  |  |  |
| Cleaning Condition |  |  |  |  |  |  |

OBSERVATIONS:

Nr. 4: Bulkhead fwd, lower stool with paint scratched and rust.

- Hold Nr. 4, bulkhead Fwd.

- Hold Nr. 4, after bulkhead

- Hold Nr. 4, star board side plate scratches and rusted

- Hold Nr. 4, Port side shell with scratches

- Hold Nr. 4, hydraulic system in order

- Hold Nr. 4, non-return valve in good conditions

- Hold Nr. 4, compression bar in good conditions

- Hold Nr. 4, roller rusted in good conditions

| HATCH & HOLD NR. 5 | G | F | P | U | Obs Nr |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hatch covers |  |  |  |  |  |  |
| Hatch coamings |  |  |  |  |  |  |
| Rubber packing Draining holes |  |  |  |  |  |  |
| Acting cleats Compression bars |  |  |  |  |  |  |
| Non return valves |  |  |  |  |  |  |
| Hydraulic Systems |  |  |  |  |  |  |
| Forward bulkhead |  |  |  |  | 4 |  |
| After bulkhead |  |  |  |  | 4 |  |
| Side/shell plating (port) |  |  |  |  | 4 |  |
| Side/shell plating (stb) |  |  |  |  | 4 |  |
| Frames, brackets and bracket connection |  |  |  |  |  |  |
| Tank top plating |  |  |  |  | 4 |  |
| Hopper tank plating |  |  |  |  | 4 |  |
| Under deck plating |  |  |  |  |  |  |
| Manholes and covers |  |  |  |  |  |  |
| Ventilation, void spaces |  |  |  |  |  |  |
| Bilges, suctions |  |  |  |  |  |  |
| Ladders |  |  |  |  |  |  |
| Air vents Sounding pipes |  |  |  |  |  |  |
| Cleaning Condition |  |  |  |  |  |  |

OBSERVATIONS:

Nr. 4: Bulkhead fwd, lower stool with paint scratched and rust.

- Hold Nr. 5, bulkhead Fwd.

- Hold Nr. 5, after bulkhead

- Hold Nr. 5, star board side plate scratches and rusted

- Hold Nr. 5, Port side shell with scratches

- Hold Nr. 5, hydraulic system in order

- Hold Nr. 5, non-return valve in good conditions

- Hold Nr. 5, compression bar in good conditions

- Hold Nr. 5, roller rusted in good conditions

IX. CARGO GEAR CONDITION

| Crane/DERRICK Nr. 1 | G | F | P | U | Obs Nr |  | CRANE/DERRICK NR. 2 | G | F | P | U | Obs Nr |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arm/Boom |  |  |  |  |  |  | Arm/Boom |  |  |  |  |  |  |
| Wires |  |  |  |  |  |  | Wires |  |  |  |  |  |  |
| Winches |  |  |  |  |  |  | Winches |  |  |  |  |  |  |
| Controls |  |  |  |  |  |  | Controls |  |  |  |  |  |  |
| Railings & Ladders |  |  |  |  |  |  | Railings & Ladders |  |  |  |  |  |  |
| Hydraulic pipes |  |  |  |  |  |  | Hydraulic pipes |  |  |  |  |  |  |
| Control Cabin Glass |  |  |  |  |  |  | Control Cabin Glass |  |  |  |  |  |  |

| Crane/DERRICK Nr. 3 | G | F | P | U | Obs Nr |  | Crane/DERRICK Nr. 4 | G | F | P | U | Obs Nr |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arm/Boom |  |  |  |  |  |  | Arm/Boom |  |  |  |  |  |  |
| Wires |  |  |  |  |  |  | Wires |  |  |  |  |  |  |
| Winches |  |  |  |  |  |  | Winches |  |  |  |  |  |  |
| Controls |  |  |  |  |  |  | Controls |  |  |  |  |  |  |
| Railings & Ladders |  |  |  |  |  |  | Railings & Ladders |  |  |  |  |  |  |
| Hydraulic pipes |  |  |  |  |  |  | Hydraulic pipes |  |  |  |  |  |  |
| Control Cabin Glass |  |  |  |  |  |  | Control Cabin Glass |  |  |  |  |  |  |

- Crane Nr. 1, external view of cabin

- Crane nr 1. General conditions

- Crane Nr. 1 arm in good conditions

- Crane Nr. 1 general conditions

- Crane Nr. 2, external view of cabin

- Crane nr 2. General conditions

- Crane Nr. 2 arm in good conditions

- Crane Nr. 2 SWL duly marked

- Crane Nr. 3, external view of cabin

- Crane nr 3. General conditions

- Crane Nr. 3 arm in good conditions

- Crane Nr. 3 external view of cabin

- Crane Nr. 4, external view of cabin

- Crane nr 4. General conditions

- Crane Nr. 4 arm in good conditions

- Crane Nr. 4 externa view of cabin

# X. BUNKER SURVEY

The following quantities were found onboard as calculated by Chief Engineer and the undersigned surveyor, from the ship’s calibration curves with due consideration to the trim of the vessel, temperature and specific gravity of oil:

Quantities found at Bunker Survey time on May 19,2026 at 11:00 hours LT (May 16, 2026 at 15:00 hours (UTC):

Consumption of Bunkers between Delivery time and Bunker survey:

Quantity of Bunkers at Delivery time on May  15, 2026 at 10:48 hours LT (May 15, 2026 at 15:48 hours UTC) at Pilot Station, Pisco, Peru:

# XI. ATTENDING PARTIES

| Name | Position | Company | On behalf of |
| --- | --- | --- | --- |
| Mr. Kang, Huaihong | Captain | MV Aquamarine SW | Owners |
| Mr. Ding , Jie | Chief Officer | MV Aquamarine SW | Owners |
| Mr. Li, Yongkang | Chief Engineer | MV Aquamarine SW | Owners |
| Mr. Ivan Meneses | Surveyor | ALS Inspection | Norden AS |
| Mr. Mauricio Lopez | Surveyor | ALS Inspection | Norden AS |

# XII. ATTACHMENTS

- Delivery Bunker Certificate

This survey was carried out without prejudice and in the interest of whom it may concern.

Surveyor ALS Inspection Chile Spa

Prepared by	: M. Lopez

Reviewed by	: A. Paredes

Validated by	: J. Lopez