FINAL REPORT

| SURVEY | MOORING ROPES INSPECTION |
| --- | --- |
| TO | GUAYACAN TERMINAL |
| FROM | ALS INSPECTION CHILE SPA. |
| CLIENT REF | N/A |
| ALS REF | VAP-2601-0183 |
| SCOPE OF SURVEY | INSPECTION OF 18 MOORING LINES. |
| VESSEL NAME | NORD AURUM |
| OWNER | NORDEN ASSET MANAGEMENT A/S |
| DISPONENT OWNER | CLASSIC MARITIME |
| CHARTERERS | CMP |
| PLACE OF SURVEY | COQUIMBO BAY |
| DATE OF SURVEY | JANUARY 15, 2026 |
| TIME OF SURVEY | FROM 08:25 HRS TO 15:30 HRS |

SUMMARY

From a total of 18 mooring lines inspected: - All of them COMPLY with the length and visual condition for use.

Definitions:

Good	: Unimpaired condition without significant wear or deviation from original strength and operating efficiency.

Fair	: Condition in which wear and tear and other deficiencies of minor nature not requiring correction or repair.

Poor	: Condition in which the adequacy of strength and/or operational efficiency is marginally below acceptable limits or is in doubt.

1.- SURVEY FINDINGS

1.1.- CHECK LIST OF MOORING ROPE N° 1

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000002 |

According OCIMF standard, letter D.2 / Cordage Institute DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 227.7 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope. |
| --- | --- |
| - Measure of Rope, 227.7 meters. | - Seal Nr. 0000002 |

1.2.- CHECK LIST OF MOORING ROPE N° 2

| ALS REFERENCE | VAP-2601-1083 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000003 |

According OCIMF standard, letter D.2 / Cordage Institute DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 220.2 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope. |
| --- | --- |
| - Measure of Rope, 220.2 meters. | - Seal N° 0000003 |

1.3.- CHECK LIST OF MOORING ROPE N° 3

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000004 |

According OCIMF standard, letter D.2 / Cordage Institute DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 229.8 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 229.8 meters. | - Seal N° 0000004 |

1.4.- CHECK LIST OF MOORING ROPE N° 4

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000005 |

According OCIMF standard, letter D.2 / Cordage Institute DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 227.0 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 227.0 meters. | - Seal N° 0000005 |

1.5.- CHECK LIST OF MOORING ROPE N° 5

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000006 |

According OCIMF standard, letter D.2 / Cordage Institute DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 228.5 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 228.5 meters. | - Seal N° 0000006 |

1.6.- CHECK LIST OF MOORING ROPE N° 6

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000007 |

According OCIMF standard, letter D.2 / Cordage Institute DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 236.7 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 236.7 meters. | - Seal N° 0000007 |

1.7.- CHECK LIST OF MOORING ROPE N° 7

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000008 |

According OCIMF standard, letter D.2 / Cordage Institute DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 220.1 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 220.1 meters. | - Seal N° 0000008 |

1.8.- CHECK LIST OF MOORING ROPE N° 8

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000009 |

According OCIMF standard, letter D.2 / Cordage Institute DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 220.2 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 220.2 meters. | - Seal N° 0000009 |

1.9.- CHECK LIST OF MOORING ROPE N° 9

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000010 |

According OCIMF standard, letter D.2 / Cordage Institute DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 220.1 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 220.1 meters. | - Seal N° 0000010 |

1.10.- CHECK LIST OF MOORING ROPE N° 10

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000806 |

According OCIMF standard, letter D.2 / Cordage Institute DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 220.6 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 220.6 meters. | - Seal N° 0000806 |

1.11.- CHECK LIST OF MOORING ROPE N° 11

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000807 |

According OCIMF standard, letter D.2 / Cordage Institute DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 221.1 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 221.1 meters. | - Seal N° 0000807 |

1.12.- CHECK LIST OF MOORING ROPE N° 12

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000808 |

According OCIMF standard, letter D.2 / Cordage Institute DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 221.3 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 221.3 meters. | - Seal N° 0000808 |

1.13.- CHECK LIST OF MOORING ROPE N° 13

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000809 |

According OCIMF standard, letter D.2 / Cordage Institute & DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 221.2 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 221.2 meters. | - Seal N° 0000809 |

1.14.- CHECK LIST OF MOORING ROPE N° 14

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000810 |

According OCIMF standard, letter D.2 / Cordage Institute & DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 221.4 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 221.4 meters. | - Seal N° 0000810 |

1.15.- CHECK LIST OF MOORING ROPE N° 15

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000801 |

According OCIMF standard, letter D.2 / Cordage Institute & DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 222.8 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 222.8 meters. | - Seal N° 0000801 |

1.16.- CHECK LIST OF MOORING ROPE N° 16

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000802 |

According OCIMF standard, letter D.2 / Cordage Institute & DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 223.1 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 223.1 meters. | - Seal N° 0000802 |

1.17.- CHECK LIST OF MOORING ROPE N° 17

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000803 |

According OCIMF standard, letter D.2 / Cordage Institute & DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 221.4 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 221.4 meters. | - Seal N° 0000803 |

1.18.- CHECK LIST OF MOORING ROPE N° 18

| ALS REFERENCE | VAP-2601-0183 |  | Date/Time | 15/01/2026 |  |
| --- | --- | --- | --- | --- | --- |
| CLIENT | ULTRAMAR |  |  |  |  |
| VESSEL/PLACE | NORD AURUM / COQUIMBO, CHILE. |  |  |  |  |
| MOORING ROPE NOMINAL LENGHT | 220 METERS | SEAL APPLIED |  |  | 0000804 |

According OCIMF standard, letter D.2 / Cordage Institute & DIRSOMAR-PROINSP/200/011/2016

| DAMAGE ITEM | GOOD | FAIR | POOR |
| --- | --- | --- | --- |
| Ext. Abrasion (max 25%) |  |  | Over 25% |
| Int. Abrasion |  |  |  |
| Cut strands |  |  |  |
| Burn/ heated parts / fusion |  |  |  |
| Sun degradation /discoloration |  |  |  |
| Fiber broken |  |  |  |
| Black stains / dirty |  |  |  |
| Inconsistent diameter |  |  |  |
| Inconsistent flexibility |  |  |  |
| Number of splices | (0) | (1- 2) | Discard 3 |

Length measured by surveyor: 223.0 meters.

Remarks: N/A

| - General view of rope. | - Diameter of Rope |
| --- | --- |
| - Measure of Rope, 223.0 meters. | - Seal N° 0000804 |

2.- COMMENTS

It is important to mention that upon our inspection of the mooring lines, these were not arranged in their entire length, even that, they were measured 2 times to achieve an average measurement.

No records of history of previous inspections of the mooring lines were shown, not did the units possess visible or recognizable identification such as plates, sleeves, seals, or permanent marks.

The mooring ropes corresponding to standard ropes. According to information received from Vessel Chief officer the vessel does not use mooring tails.

Visual inspection of mooring ropes was carried out with natural light.

The inspected mooring lines were also properly marked with a color, green for accepted lines.

From a total of 18 mooring lines inspected, all of them comply with the conditions for use.

This survey was carried out without prejudice and in the interest of whom it may concern.

Surveyor

ALS INSPECTION CHILE SPA

Prepared by  : V. Alfaro

Reviewed by : C. Alvear

Validated by :  J. Lopez