| # INSPECTION OF CONTAINER STRUCTURAL CONDITION BEFORE STUFFING OPERATION |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ALS INSPECTION REFERENCE VAP-2604-1325 INSPECTION ON BEHALF OF Bertling Logistics Chile S.A. REQUESTED BY Mr. Ariel Valdenegro CONTAINER 20” Nr. TCIU 400248-0 DATE MANUFACTURED August 2024 ALS SURVEYOR Alexander Luzardo I. PLACE OF INSPECTION Servicontainers S.A., Placilla, Valparaiso - Chile ATTENDANCE DATE April 02, 2026 |  |  |  |  |  |  |
| Question |  | Answer |  |  |  |
| CONTAINER COMPLIANCE CHECKLIST FOR IMPORT/EXPORT OF EXPLOSIVES |  | Container Selection |  |  |  |
| ### 1.- General Requirements |  |  |  |  |  |
| 1.1.- Non-vented general purpose ISO container |  | Compliance |  |  |  |
| 1.2.- Valid CSC approval plated fitted or equivalent (ACEP No. legible) |  | Compliance |  |  |  |
| 1.3.- Owners serial No. On CSC plate and container |  | Compliance |  |  |  |
| ### 2.- Doors |  |  |  |  |  |
| 2.1.- Only two openings |  | Compliance |  |  |  |
| 2.2.- Each door with two locking bars (ie. 4) |  | Compliance |  |  |  |
| 2.3.- Door hinges serviceable and operative |  | Compliance |  |  |  |
| 2.4.- Doors capable of being locked |  | Compliance |  |  |  |
| 2.5.- Gaskets and seals in good condition |  | Compliance |  |  |  |
| 2.6.- Weather tight |  | Compliance |  |  |  |
| ### 3.- External |  |  |  |  |  |
| 3.1.- Top and bottom side rails |  | Compliance |  |  |  |
| 3.2.- Top and bottom end rails |  | Compliance |  |  |  |
| 3.3.- Door sill and header |  | Compliance |  |  |  |
| 3.4.- Corner posts |  | Compliance |  |  |  |
| 3.5.- Corner fittings |  | Compliance |  |  |  |
| 3.6.- Side and end walls weathertight |  | Compliance |  |  |  |
| ### 4.- Internal |  |  |  |  |  |
| 4.1.- Free of residue from previous cargo |  | Compliance |  |  |  |
| 4.2.- Free from rust and scale |  | Compliance |  |  |  |
| 4.3.- In clean condition |  | Compliance |  |  |  |
| 4.4.- Free of projections or protrusions |  | Compliance |  |  |  |
| Remark: The lining specified is not required where all of the explosives in the container are in packaging’s having substantial wooden outside surfaces or all the explosives are of Blasting Type E (UN0241, UN0332), in which case non lined steel containers may be used |  |  |  |  |  |
| References: IMDG Section 7.4.6.4.2, CSC Code, Australian Explosives Code Section 6.3. |  |  |  |  |  |
| CONCLUSION |  | APPROVED |  |  |  |
| CONTAINER SEAL |  | ALS 2921442 |  |  |  |
| Date & Time of Survey |  | April 02, 2026 – 16:33 to 16:40 Hrs. |  |  |  |
| This survey was carried out without prejudice and in the interest of whom it may concern. |  |  |  |  |  |
| SURVEYOR | Alexander Luzardo I. |  | April 02, 2026 |  |  |

## PHOTOGRAPHIC SET

| - General view of unit 20´ | - CSC plate |
| --- | --- |

| - Side panel in good condition. | - Top riel and front panel in good condition. |
| --- | --- |

| - Tight test. | - Inside without damage. |
| --- | --- |

| - Container floor in good conditions. | - Seal employed N° ALS 2921442 approved. |
| --- | --- |